| Schedule 1 - Load Management Protocol Data Sharing Agreement |  |
| --- | --- |
| Date | [Date of contract] |
| EDB Contact | Name (email) Name (email) |
| Version | 2.0 |
| Abstract | [ ] (the distributor) and [ ] (the retailer) agreed to the ENA shared load management protocol. This data request provides the distributor with the information required under the load management protocol. |

# Version Control

| Version | Amendment Date | Comments |
| --- | --- | --- |
| 1.0 | March 2026 | Version of the document associated with V1 of ENA Load Management Protocol. |

# Context

This is an accompanying document to the ENA shared load management protocol. All connection points where a trader has controllable load should be included in this data request.

# Requirements

As this is a customer information request, the management and utilisation of the data would comply with the Registry Customer Information protocol (v11), with the following details:

- Destination address: [as defined by each EDB]
Host name: sftp.electricityregistry.co.nz 
Directory: 	/EIEPin

- Data security: As stipulated in the protocol.

- Snapshot/Incremental: Snapshot of daily data. Refer to Table A.2 for more information.

- Data update frequency: Weekly – Uploaded to the destination folder every Monday before 23:59 PM NZST. Refer to Table A.2 for more information.

- Data file type: Comma-separated value (.CSV) file

- Data file naming convention: 
<Recipient participant identifier>_E_<EDB Identifier>_LMPSCRIPT_YYYYMM_YYYYMMDD_hhmm.csv

where `YYYYMM` denotes the year-month of the snapshot, `YYYYMMDD` denotes the snapshot date, and `hhmm` denotes the snapshot time.

Example: Daily snapshot of ICPs within control groups extracted from the retailer system on Monday, 24th Feb. 2025, 11:55 PM will have a data file named as follows:

“XXXX_E_XXXX _LMPSCRIPT_202502_20250224_2355.csv”

- Data columns: Refer to Table A.1.

# Appendix

Table A.1: Data Dictionary

| Column Name | Format | Mandatory/Optional/ Conditional | Description | Example |
| --- | --- | --- | --- | --- |
| connection_point | Char 15 | Mandatory | Unique identifier for an installation control point (ICP). | 012345678AB12X |
| meter | Char 15 | Mandatory | Meter serial ID of the meter installed at the ICP. | 999999999 |
| shared_control | Char 1 | Mandatory | If the controllable load at the ICP is also controllable by the distributor Y = shared, if it is not N = not shared, U = unknown | Y |
| controllable_load_kW | Float | Mandatory | Total controllable load in kW at the ICP. This should be the nameplate, undiversified controllable load. | 3.0 |
| energy_storage_kWh | Float | Mandatory | Total available storage in kWh at the ICP. This should be the nameplate, fill capacity storage. Devices with no storage will show 0 kWh. Can be approximation. | 11.5 |
| control_priority | Char 1 | Mandatory | Either 1,2 or 3 according to the priority classification | 1 |
| scheduled_daily_off_TPs | Char 185 | Conditional | If the controllable load is on a fixed schedule, like smart meter controlled hot water with calendar control, provide a comma separated list of the daily trading periods when the controllable load is scheduled to be OFF (not drawing power from the grid). *The list must be enclosed by double quotation marks (e.g. “1, 2, 3”). Trading periods are 1-48 as defined by the Electricity Authority here: https://www.ea.govt.nz/data-and-insights/tools-and-apis/ If left blank the EDB will assume that the controllable load is dynamically responding to control signals from the Trader and will choose how to evaluate the potential network impact. | “15, 16, 17, 18, 19, 20, 21, 22, 35, 36, 37, 38, 39, 40, 41, 42” |
| random_restore_minutes | Integer | Mandatory | Defined period in minutes during which controllable load is set to randomly restore following a controlled OFF period for the purpose of avoiding synchronized peaks | 30 |

Tables A.2: Example scenarios in a 7-day period

| Connection _point | meter | shared_control | Controllable _load_kW |  | energy_storage_kWh |  | control_priority | scheduled_daily_off_TPs | random_restore _minutes |  | Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | 1 | Y | 3.0 |  | 10 |  | 1 | “15, 16, 17, 18, 19, 20, 21, 22, 35, 36, 37, 38, 39, 40, 41, 42” | 10 |  | Example snapshot of hot water, where EDB has ripple control and where Trader has scheduled hot water to be turned off daily via a schedule from 0700-1100 and 1700-2100. |
| B | 2 | Y | 3.0 |  | 10 |  | 1 | “15, 16, 17, 18, 19, 20, 21, 22, 35, 36, 37, 38, 39, 40, 41, 42” | 10 |  |  |
| C | 2 | Y | 3.0 |  | 10 |  | 1 | “15, 16, 17, 18, 19, 20, 21, 22, 35, 36, 37, 38, 39, 40, 41, 42” | 10 |  |  |
| D | 1 | Y | 3.0 |  | 5 |  | 1 | “15, 16, 17, 18, 19, 20, 21, 22, 35, 36, 37, 38, 39, 40, 41, 42” | 10 |  |  |
| E | 2 | Y | 3.0 |  | 5 |  | 1 | “15, 16, 17, 18, 19, 20, 21, 22, 35, 36, 37, 38, 39, 40, 41, 42” | 10 |  |  |
| F | 1 | Y | 3.0 |  | 5 |  | 1 | “15, 16, 17, 18, 19, 20, 21, 22, 35, 36, 37, 38, 39, 40, 41, 42” | 10 |  |  |
| Connection _point | meter | shared_control |  | Controllable _load_kW | energy_storage_kWh |  | control_priority | scheduled_daily_off_TPs | random_restore _minutes |  | Explanation |
| A | 1 | N |  | 11.0 | 0 |  | 2 |  | 30 |  | Example snapshot of controlled EV chargers with no EDB control and no scheduled control |
| B | 1 | N |  | 7.0 | 0 |  | 2 |  | 30 |  |  |
| C | 1 | N |  | 11.0 | 0 |  | 2 |  | 30 |  |  |
| D | 1 | N |  | 7.0 | 0 |  | 2 |  | 30 |  |  |
| E | 1 | N |  | 7.0 | 0 |  | 2 |  | 30 |  |  |
| F | 1 | N |  | 7.0 | 0 |  | 2 |  | 30 |  |  |